Connecting TFT 1.8 display to NodeMcu ESP8266:

1st of all connect the tft display through the direction:

- D8 -> CS

- D7 -> SDA

- D5 -> CSK

- D4. -> A0

- D3 -> RESET

- Vcc -> 3.3V

- Led -> 3.3V

- GND -> GND

Circuit Connection:

Then upload the code :

* ForbiddenBit.com * * NodeMCU and Display ST7735 */ // -------- Libraries required to use the Display --------------- #include <Adafruit_GFX.h> #include <Adafruit_ST7735.h> #include <SPI.h> // ----------- Display pins -------------------------------------- #define TFT_CS 15 #define TFT_RST 0 #define TFT_DC 2 void setup() { Adafruit_ST7735 tft = Adafruit_ST7735(TFT_CS, TFT_DC, TFT_RST); //Those things are for the display tft.initR(INITR_GREENTAB); // -------- TEST TEXT ------------------------------------------- tft.fillScreen(ST77XX_BLACK); tft.setTextWrap(false); tft.setCursor(45, 5); //Horiz/Vertic tft.setTextSize(2); tft.setTextColor(ST77XX_YELLOW); tft.print("USA"); tft.setCursor(15, 25); //Horiz/Vertic tft.setTextSize(1); tft.setTextColor(ST77XX_WHITE); tft.print("COVID-19 Tracker"); tft.setCursor(5, 45); //Horiz/Vertic tft.setTextSize(2); tft.setTextColor(ST77XX_BLUE); tft.print("Cases:"); tft.setCursor(10, 65); tft.print("929,637"); tft.setCursor(5, 93); //Horiz/Vertic tft.setTextSize(2); tft.setTextColor(ST77XX_GREEN); tft.print("RECOVERED:"); tft.setCursor(15, 115); tft.setTextColor(ST77XX_WHITE); tft.print("110,504"); tft.fillRect(0, 138 , 48, 10, ST77XX_WHITE); tft.setCursor(7, 140); //Horiz/Vertic tft.setTextSize(1); tft.setTextColor(ST77XX_BLACK); tft.print("DEATHS: "); tft.setCursor(55, 140); tft.setTextColor(ST77XX_WHITE); tft.print("52,838"); } void loop() { }

After uploading the Code we can see the display: